### CONTENTS

Page

0	Foreword								      1

1.0	Scope									      2

2.0	Definitions								      2

3.0	Composition and Quality Factors					      4

Table 1:    Natural Mineral Water				      6

Table 2:    Spring Water					      7

Table 3:    Purified/Prepared Waters				      9

4.0	Requirements for Hygiene in Collecting,				     10

Processing and Marketing of Packaged Water

5.0	Packaging								     11

6.0	Labelling								     11

7.0	Quality Assurance							     15

8.0	Sampling								     15

Table 4:    Number of containers to be selected		     16

for Testing

9.0	Test methods								     16

10.0	Conformity								     40

Appendix I Recommended Test Methods				     41

0		FOREWORD

0.1		This standard has been prepared to set levels of quality and safety for packaged water, produced and traded in Grenada.

0.2		In Grenada water from natural springs or sources are being packaged and sold.  There are many underground reservoirs that are capable of exploitation, which have not been exposed to any pollution, and can be used to satisfy a growing demand for naturally pure water, or water containing certain minerals.  Purified waters from other sources are also entering the trade, and it is now desirable to set standards for the production and promotion of such products.

0.3		This standard is identical to the Caricom Community Standard – Specification for Packaged Water.

0.4		In preparing this standard guidance was obtained by study of the following documents:

(a)   The Food and Drugs Regulations, Canada (1980-1988).

Division 12;

- The Food and Drug Regulations, United States, 1996, title 21, sub-

part 103.35 and Part 129 of the Code of Federal Regulations;

(c)   Codex Alimentarius Standard CODEX STAN 108-1981,

Rev. 1-1997;

(d)   Codex Stan 227-2001 General Standard for Bottled/Packaged

Drinking Waters (Other than Natural Mineral Waters);

(e)   Guide to Good Bottled Water Standard, BSDA, 1995;

(f)   Beverages: Technology, Chemistry and Microbiology, Allan H.

Varnan and James P. Sutherland, Vol. 2;

(g)   ISO Standard Compendium, Environmental – Water Quality, vol. 3 -

Physical, Biological and Microbiological Methods, First Edition;

(h)   Statutory Instruments 1999 No. 1540 Food: The Natural Mineral

water, Spring Water and Bottled Drinking water Regulations 1999

(United Kingdom).

0.5	This standard will be reviewed, and may be revised from time to time to allow for changes in manufacturing technology or consumer preferences.

0.6		This standard was adapted as a Grenadian Standard in May 2003 by the Standards Council of Grenada.

- SCOPE

  - This standard specifies requirements for the purity, treatment, bacteriological acceptability, packaging and labelling of all packaged waters that are prepackaged for sale and used as beverages or in foods.

  - This standard does not apply to water distributed by public water supply system, to the carbonated beverage soda water, or to packaged water sold for purposes other than as a beverage.

  - This standard should be used in conjunction with the Caricom Code of Hygienic Practice for the Collecting, Processing and Marketing of Packaged Water CCS: 2000.

- DEFINITIONS

  - “Packaged Waters” means other than natural mineral waters, are waters for human consumption and may contain minerals, naturally occurring or intentionally added; may contain carbon dioxide, naturally occurring or intentionally added; but shall not contain sugars, sweeteners, flavourings or other foodstuffs.

  - “Waters Defined by Origin” shall share the following characteristics:

- they originate from specific environmental resources without passing

through a community water system,

- precautions taken within the vulnerability perimeters to avoid any

pollution of, or external influence on, the chemical, microbiological and physical qualities of water at origin,

- collecting conditions which guarantee the original microbiological

purity and essential elements of their chemical make-up at origin.

  - Spring Water means the name of water derived from an underground formation from which water flows naturally to the surface of the earth may be spring water. Spring water shall be collected only at the spring or through a borehole tapping the underground formation feeding the spring. There shall be a natural force causing the water to flow through the surface through a natural orifice.  The location of the spring shall be identified.  Spring water collected with the use of an external force shall

be from the same underground stratum as the spring, as shown by a

measurable hydraulic connection using a hydrogeologically valid method between the bore hole and the natural spring, and shall have all the natural properties, before treatment and be of the same composition and quality, as the water that flows naturally to the surface of the earth.

  - Natural Mineral Water is a water clearly distinguishable from ordinary drinking water because: it is characterized by its own content of certain mineral salts and their relative proportions and the presence of trace elements or of other constituents; it is obtained directly from natural or drilled sources from underground water bearing strata for which all possible precautions should be taken within the protected perimeters to avoid any pollution of, or external influence on, the chemical and physical qualities of natural mineral water; of the constancy of its composition and the stability of its discharge and its temperature, due account being taken of the cycles of minor natural fluctuations; it is collected under conditions which guarantee the original microbiological purity and chemical composition of essential components; it is packaged close to the point of emergence of the source with particular hygienic precautions; it is not subjected to any treatment other than those permitted by this standard.

  - Potable Water means water that is naturally suitable or artificially made

suitable for human consumption and as such is free from disease causing

germs or microorganisms.

2.6		Prepared Waters do not comply with all the provisions set for waters defined by origin.  They may originate from any type of water supply.

2.7		Purified Water means potable water that is obtained from an underground source or other suitable sources (including public water supply), and does not contain any concentrations of inorganic substances in excess of 500 ppm and which confer beneficial physiological effects.

2.8		A Public Water Supply System means a source of potable water operated by a public utility, a company or other body, using distribution through pipelines or tank-wagons.

2.9		UV radiation means the process of subjecting water to radiation between the wavelengths 220 and 300 nanometers for the purposes of disinfection.  For this process 90% of the radiation must be of the wavelength 254 nanometers.

2.10		Reverse Osmosis refers to the use of membrane filters to remove dissolved solids from water.

2.11		De-ionization means the process by which water is passed through ionic exchange resins for the removal of dissolved minerals.

(Note 1: Plants shall demonstrate, on request, to appropriate regulatory

officials, using a hydrogeologically valid method, that an appropriate

connection exists between the natural orifice of the spring and the bore

hole).

2.12		Distillation means the process of heating water and condensing it in such a manner as to remove dissolved minerals from the water.

3.0		COMPOSITION AND QUALITY FACTORS

3.1		MODIFICATION, TREATMENT AND HANDLING OF PACKAGED WATERS

    - Natural Mineral Water

3.1.1.2	Natural Mineral water shall be obtained from an underground aquifer that

is not polluted by agricultural, domestic, industrial, or other wastes.

3.1.1.3 	Treatment of these waters may be carried out only on condition that the

mineral content of the water is not modified in its essential constituents which give the water its properties.  Natural mineral water may be treated by the following processes:

- decantation and/or filtration to remove suspended or undissolved

matters and unstable constituents such as compounds containing iron,

manganese, sulphur and arsenic.

- aeration with clean, filtered air.

# 3.1.2                Spring Water

3.1.2.1             Spring water may be treated by processes that remove unstable or

undissolved matter, influence the microbiological population and the physical and chemical characteristics of the water.  Such treatments shall be applied on condition that, when the water is sampled as in Section 12 of this standard, the characteristics of the original water comply with the provisions of Table 2 and Sections 3.2, 3.4, 3.5 and 3.6 of this standard.

These processes include:

(a)  decantation to remove solids

- filtration to remove particles of suspended matter

- aeration with clean filtered air

- reverse osmosis

- ion exchange

- precipitation

- ultra/micro-filtration and activated charcoal filtration

- ozonation

- ultra-violet radiation.

3.1.3	Purified Water (Prepared Waters)

3.1.3.1	Purified waters may be subjected to treatments that modify the

microbiological, physical and chemical characteristics of the water.  Such treatments shall be applied on condition that, when the water is sampled as in Section 8 of this standard, the characteristics of the water conform with all the provisions of Table 3 and Sections 3.2 and 3.6 of this standard.

      - Purified water/prepared water shall be obtained by applying any one of the

following processes:

- decantation;

- filtration;

- clarification by using chemical agents and may be treated with:

- chlorine or a source of chlorine (with excess chlorine being removed by aeration);

(e)  activated carbon to remove chlorine, odours or flavours.

In addition, purified/prepared water may

- be distilled;

- have added fluoride, or ozone;

- be demineralised, so that inorganic substances are reduced below 10 parts per million;

- be carbonated;

- be treated by reverse osmosis;

- treated with ultra violet radiation;

- be treated with ozone prior to packaging.

  - MICROBIOLOGICAL REQUIREMENTS

    - Mineral water, spring water and purified water, when sampled and tested

within 12 hours of packaging, as in Sections 8 and 9, shall contain:

(a)   no coliform bacteria in 250 mL at 37 0C   and 44.5 0C;

- no faecal Streptococci in 250 mL;

- no Pseudomonas aeruginosa in 250 mL;

- (i)   no more than 100 aerobic bacteria per millitre at 22 0C in 72

hours;

(ii)   no more than 20 aerobic bacteria per mL at 37 0C in 24 hours.

  - LIMITS ON CERTAIN SUBSTANCES

    - Mineral water, spring water or purified/prepared waters, when sampled as

in Section 8 and tested by the methods mentioned in Section 9, shall not contain the substances named in the respective tables in concentrations exceeding the limits specified.

### TABLE 1

MAXIMUM CONCENTRATIONS OF CERTAIN SUBSTANCES IN NATURAL

MINERAL WATER IN mg/L (parts per million)

-----------------------------------------------------------------------------------------------

(1)                                                                            (2)

Substance                                                            Maximum Concentration

-------------------------------------------------------------------------------------------------

Aluminium						     0.2

Antimony						     0.005

Arsenic							     0.01, calculated as As

Barium							     0.7

Cadmium						     0.003

Chromium (VI)						     0.05 calculated as total Cr.

Copper							     1.0

Lead							     0.01

Manganese						     0.5

Mercury							     0.001

Nickel							     0.02

Selenium						     0.01

Thallium							     0.002

Zinc							     5.0

Borate							     5   calculated as B

Cyanide							     ------

Fluoride							     See section 6.6.1 (d)

Chloride							     250

Nitrate							     50 calculated as NO3-

Nitrite							     -------

Sulphide							     0.05 calculated as H2S

Ra226   +   Ra228					     15pCi/L

------------------------------------------------------------------------------------------------------------

____________________

1  Any increase in the total viable colony count of the water between 12 hours after packaging and the time

of sale shall not be greater than that normally exposed.

    - Natural mineral waters, when sampled as above, shall not contain the

following substances in amounts above the limits quantified in accordance with Codex Alimentarius, Volume 13:

–	Surface active agents

–	Pesticides and PCBs

–	Mineral Oil

–	Polynuclear aromatic hydrocarbons

TABLE 2 A

MAXIMUM CONCENTRATIONS OF CERTAIN SUBSTANCES IN

SPRING WATER in mg/L (parts per million)

---------------------------------------------------------------------------------------------------------

(1)                                                                                          (2)

Substance                                                                      Maximum Concentration

---------------------------------------------------------------------------------------------------------

Magnesium							       50

Nitrogen							        1

Potassium							       12

Sodium							       150

Sulphate							       250

Ammonium (ammonia					       0.5

and ammonium ions)

Nitrate								       50

---------------------------------------------------------------------------------------------------------

TABLE 2 B

MAXIMUM CONCENTRATION OF CERTAIN SUBSTANCES

IN SPRING WATER IN µg/l (parts per billion)

----------------------------------------------------------------------------------------------------------

(1)                                                                                    (2)

Substance                                                                   Maximum Concentration

----------------------------------------------------------------------------------------------------------

Aluminium							      200

Iron								      200

Manganese							      50

Copper								      2000

Zinc								      5000

Phosphorus							      2200

Fluoride								      1500

Silver								      10

Arsenic								      10

Cadmium							      5

Chromium							      50

Cyanide								      50

Mercury								      1

Nickel								      50

Selenium							      10

Antimony							      10

Lead								      10

Dissolved or emulsified						      10

hydrocarbons; mineral oils

Phenols								      0.5

TABLE 3

MAXIMUM CONCENTRATIONS OF CERTAIN SUBSTANCES IN

PURIFIED WATER IN mg/L (parts per million)

-------------------------------------------------------------------------------------------------------

(1)                                                                            (2)

Substance                                                         Maximum Concentration

--------------------------------------------------------------------------------------------------------

Aluminium						         0.2

Antimony						         0.005

Arsenic							         0.05

Barium							         0.7

Cadmium						         0.003

Chromium (VI)						         0.05

Copper							         1.0

Iron							         0.03

Lead							         0.01

Manganese						         0.5

Mercury							         0.001

Nickel							         0.02

Thallium							         0.002

Selenium						         0.01

Zinc							         3.0

Borate							         30 calculated as H3BO3

Cyanide							         0.07

Fluoride							         1.5 calculated as F-

Organic matter						         3 calculated as O2

Chloride							         250

Nitrate							         45 calculated as NO3-

Nitrite							         3

Sulphate							         250

----------------------------------------------------------------------------------------------------------

  - CONTAMINANTS

    - Packaged water shall not contain

- cyanide ion;

- nitrite ion;

- detectable residues of pesticides, such as:

(i)   Organochlorines e.g. endrin, lindane,

toxaphene, 2-4-D, 2, 4, 5 - TP;

(ii)   Organophosphates e.g. pirimiphos - ethyl, ethoprop,

diazion, malathion, glyphosate;

(iii)   Carbamates e.g - carbofuran, oxamyl, propoxur;

(iv)   Bipyridinium salts e.g - paraquat, diquat.

- trihalomethanes;

(e)   polycyclic aromatic hydrocarbons.

3.5 		Addition 0f Minerals

3.5.1		Any addition of minerals to packaged waters must comply with the

provisions outlined in this standard and the CODEX General Principles for the Addition of Essential Nutrients to Foods (CAC/GL 9-1987).

4.0	REQUIREMENTS FOR HYGIENE IN COLLECTING, PROCESSING AND MARKETING OF PACKAGED WATER

  - The products covered by the provisions of this standard shall be prepared

in accordance with the Recommended International Code of Practice for the Collecting, Processing and Marketing of Natural Mineral Waters (CAC/RCP 33-1985) and the Code of Hygienic Practice for Bottled/Packaged Drinking Waters (other than Natural Mineral Waters) (CAC/RCP 48-2001).

5.0		PACKAGING

5.1		Mineral water, spring water and purified water shall be packed in

hermetically sealed retail containers, which are suitable for preventing the possible adulteration of the water.  Retail containers and closures shall be made of non-toxic materials that will not contaminate the water or affect its flavour, and shall be designed to withstand stresses that may be experienced in bottling, handling, transport and storage.

  - The packages used for containing water for sale shall be made of glass,

polyethyleneterephthalate (PET) or other suitable food grade plastic material.

  - At regular intervals, samples of unfilled containers and closures shall be

sampled at the point of filling, and tested bacteriologically for coliform organisms.  At least four containers and four closures shall be taken, and the packaging process shall be deemed acceptable if:

- no coliform organisms are found and

- not more than 1 bacterium/mL of container capacity, or not more than

1 bacterium/cm2 of container surface is found.

  - Retail containers shall be protected during transport by suitable shipping

cartons or crates.  If crates are reusable they shall be inspected before re-use and cleaned as may be necessary to minimize risk of contamination of the product.

  - Closures shall be so designed as to prevent contamination and to show

signs of any tampering after filling and sealing the containers.

6.0		LABELLING

6.1		The labeling on retail packages of mineral water, spring water, and

purified water shall be in the English language, clearly and prominently displayed, and readily discernible under customary conditions of purchase and use.

  - Information presented in other languages shall be clearly separated from

that in English.

  - General Labelling

    - Labels on retail containers of mineral water, spring water, and purified

water shall carry the following information:-

- the brand name or trade name, if any;

(b)   the name and postal address of the manufacturer or bottler, or of the

person controlling the brand name, together with an adequate street

address;

(c)   the name of the country of origin;

(d)   the average net contents in the container, declared by volume in the

metric (Systeme Internationale) system of units;

- the date of filling of the container, and/or a lot number or batch

number;

(f)   the words “Best Before” followed by a date up to and including that

which the water can reasonable be expected to retain its specific properties if stored properly

(g)   recommended storage conditions, if the integrity of the water depends

on the storage conditions.

  - Purified Water

    - Labels on retail containers of purified water shall include the following

Information in addition to that required by 6.3:-

- the product name, “Purified Water” or “Water” or “Pure Drinking Water” or any other description which may enhance the marketability of the package provided such description is not false or misleading, which may be modified by the words:

(i)   “Distilled,” when treated by distillation;

(ii)   “Demineralised,” where the mineral content has been reduced by

other means than distillation;

(iii)   “Carbonated” or “sparkling” where carbon dioxide has been

added;

(iv)   “Non carbonated” or “non sparkling” or “still” where there is no

visible and spontaneous release of carbon dioxide under normal

conditions of temperature and pressure when the package is

opened.

- a statement of the total dissolved solids content of the packaged

water, for waters defined by origin, the chemical composition that confers the characteristics to the product may be declared on the label;

- an indication of the method used in treatment except where the water

has been

(i)   chlorinated, followed by removal of chlorine and chlorinating

agent;

(ii)   decanted;

(iii)   filtered, or

(iv)   an ingredient declared on the label has been added;

(v)   treated by reverse osmosis.

  - Spring Water

    - Labels on retail containers of spring water shall carry the following

information in addition to that required by 10.3:-

- a statement of the total dissolved mineral salt content in mg/L or parts

per million (ppm);

- the total fluoride content in mg/L or parts per million (ppm);

- if carbon dioxide has been added, the product shall be described as “carbonated spring water.”

    - Labels on retail containers of spring water may also include;

- a statement of the results of chemical analysis of the water of the

source, or as packaged in the container;

- “low sodium,” if the sodium ion content is less than 20 parts per

million (ppm);

- “sodium free,” if the sodium content is less than 5 ppm.

  - Mineral Water

    - Labels on retail containers of mineral water shall carry the following

information in addition to that required by 6.3:-

- the name of the product;

- the geographical location and the name of the source shall be declared;

- a statement of the total dissolved mineral salt content in mg/L or parts per million (ppm);

- if carbon dioxide has been added that was not present when emerging

from the source, or in amounts greater than that originally present, the product shall be described as “carbonated mineral water,” and

- a statement of the results of chemical analysis of the water of the

source, or as packaged in the container.

    - Labels on retail containers of mineral water may also include:-

- a statement of the process used in treatment, as in 3.1 and the results of the treatment shall be declared on the label;

(b)   the word “alkaline”, where the content of bicarbonate ion, HCO3-,

exceeds 600 ppm;

(c)   the word “saline” where the content of sodium chloride, NaCl,

exceeds 1000 ppm;

- the words “low in sodium” where the content of sodium ion, Na+ is

less than 20 ppm;

- the words “contains fluoride” where the content of fluoride ion, F-,

exceeds 1 ppm;

- the words “contains iron” where the content of iron, Fe2+, exceeds 1

ppm;

- the words “contains iodide”, where the content of iodine ion, I-,

exceeds 1 ppm;

- the words “may be diuretic,” where the content of total dissolved solids exceeds 1000 ppm, or the level of bicarbonate ions is greater than 600 ppm;

(i)   the words “may be a laxative” where the product contains greater than

600 ppm of sulphate other than calcium sulphate.

    - Where a source of mineral water has been inspected, sampled, tested and

approved by an official agency that is concerned with public health, a statement of such approval may be included on the label.

  - Statements not to be used in Labelling or Advertising

    - No statement or pictorial device shall be used on a label of a retail

container of mineral water, spring water, or purified water which may mislead the consumer as to its nature, origin, composition, or properties.

6.7.2		Trade or brand names referring to mineral water or spring water shall not

include a name of a location or community unless the source is located within that location or community.

6.7.3		The trade or brand name referring to purified water shall not include a

reference to a geographical feature, location or community.

    - No claims for medicinal effects (whether preventative, nutritive,

alleviative, or curative) shall be made in labels or advertisements of mineral water, spring water, or purified water, other than those allowed  by 10.6.2.

7.0		QUALITY ASSURANCE

7.1		To be eligible for a license to use the Caricom Standard Mark, the

manufacturer shall operate a quality assurance system conforming to the general requirements of International Standard ISO 9001-2000, using adequate staff, sampling procedures, testing equipment, and record-keeping as approved by the competent authority.

7.2		An approval quality assurance system may include sampling or test

procedures suited to routine or continuous production that differ from

those mentioned in sections 12 and 13.

7.3		It is recommended that a Hazard Analysis Critical Control Point (HACCP)

system be used as part of the quality assurance procedures.

8.0		SAMPLING

8.1		Where a lot or shipment of bottled water is to be examined for conformity

to this standard, it is recommended that a sample including a number (n) of retail containers should be taken at random from the total number (N) of retail containers in the lot or shipment in accordance with columns 1, 2 and 3 of Table 3.

TABLE 4

Number of Containers to be Selected for Testing

(1)				(2)			(3)

Number in the Lot			For			For

Bacteriological	       Chemical

Tests		           Tests

N				n		             n

---------------------------------------------------------------------------------------------------

Up to 1300				3			 9

1301 to 3200				6			12

3201 to over				9			15

---------------------------------------------------------------------------------------------------

  - The contents of retail containers sampled for chemical testing may be

combined in a clean dry container, and shall be sealed and marked with identifying information (place, date and time of sampling, markings on the lot, name of sampler).

  - Retail containers sampled for bacteriological testing shall be kept separate

and each sealed and marked with identifying information.

  - The samples shall be delivered and tested as soon as possible after

collection, with precaution against contamination or deterioration.

9.0		TEST METHODS

9.1		It is recommended that the following test methods be used in connection

with this standard.  These methods are based on that published by the International Organization for Standardization (ISO), ISO Standard Compendium, Environmental - Water Quality, Vol. 3 – Physical, Biological and Microbiological Methods.

  - Basic materials

For the preparation of the medium, use ingredients of uniform quality and chemicals of analytical grade; alternatively, use an equivalent dehydrated complete medium and follow the manufacturer’s instructions.

For making media, use glass-distilled or deionized water free from substances which might inhibit growth under the conditions of the test.

  - Dilutions

For making the dilutions use one of the diluents given in ISO 8199.

  - Sampling

Take the samples of water in accordance with the instructions given in Section 12 of this document.

  - COLONY COUNT BY INOCULATION IN OR ON A NUTRIENT

#### AGAR CULTURE MEDIUM

    - Culture Medium and Diluents

9.5.2		Yeast extract agar

Tryptone………………………………………                              6g

Dehydrated yeast extract………………………                             3g

Agar, powder or in pellets………………………                           12g

(according to gel strength)

Water…………………………………………….                          100 ml

Add the ingredients, or the complete dehydrated medium, to the water and

dissolve by heating.  Adjust the pH if necessary so that after sterilization it

will be 7.2 ± 0.2 at 25 0C.

Distribute volumes of 15 mL in tubes, bottles or other containers.  For

storage in large volumes, use containers up to 500 mL capacity.  Sterilize

in the autoclave at 121 0C for 15 min.

For use, melt the medium, allow to cool and maintain it at 45 ± 1 0C using

the water bath.

9.5.3		Apparatus and glassware

Usual microbiological laboratory equipment, and in particular.

    - Apparatus for sterilization by steam (autoclave)

9.5.5 		Incubators capable of maintaining a temperature of 37 ± 2 0C.

9.5.6		Glass or plastic Petri dishes with a diameter of 90 mm or 100 mm.

9.5.7	Water bath, or similar apparatus capable of maintaining a temperature of 45 ± 1 0C.

    - Colony-counting equipment with a method of illumination against a dark

background, a hand-lens (optional) and preferably a mechanical or electronic digital counter.

    - Procedure

    - Preparation and Inoculation

Carry out preparation of the sample, make dilutions and inoculation of culture media, in accordance with ISO 8199.

For pour plates, place the test volume in the Petri dish, add the molten medium and mix carefully by gentle rotation; allow the medium to set.  For spread plates, place the test volume on the dry surface of the agar medium and distribute it over the surface with a sterile glass rod; allow the inoculum to be absorbed.

Inoculate at least two plates for each test volume at each temperature.

    - Incubation and examination

Invert the plates and incubate one set at 37 ± 1 0C for 24 ± 1 h or 48 ± 4 h; include the other set of plates at 22 ± 1 0C for 72 ± 4 h.  Examine the plates as soon as they are removed from the incubators; if this is not possible, store them at 4 0C and examine them within 24 h.  Reject any plates with confluent growth.

9.5.12		Counting of Colonies

Count the colonies present in or on each plate, if necessary with

magnification and the aid of a counting device.

Determine the average number of colonies from the pairs of plates from

each dilution, each plate ideally containing between 24 and 300 colonies.

For each temperature of incubation, calculate the estimated number of

colony-forming units present in 1 mL of the sample.

Alternatively, if more than one pair of dilutions yields counts of between

25 and 300 colonies, then determine the weighted mean according to the

formula given in 8.4 of ISO 8199.  From these values estimate for each

temperature of incubation the number of colony-forming units present in

1 mL of the sample.

9.5.13		Expression of results

Express the results as the number of colony-forming units per milliliter of

the sample for each temperature of incubation.

If there are no colonies in or on the plates inoculated with test volumes of

the undiluted sample, express the results as less than 1 colony-forming

unit per milliliter.  If there are more than 300 colonies on the plates

inoculated with the highest dilutions used, express the results as

approximate only.

9.6	DETECTION AND ENUMERATION OF FAECAL STREPTOCOCCI

#### Part 1: Method of enrichment in a liquid medium

    - Culture media and reagent

Warning - All selective media described in this part of the document contain sodium azide.  As this substance is highly toxic and mutagenic, precautions should be taken to avoid contact with it, especially by the inhalation of fine dust during the preparation of commercially available dehydrated complete media.  Azide-containing media should not be mixed

with strong inorganic acids, as toxic hydrogen azide (HN3) may be produced.  Solutions containing azide can also form explosive compounds when in contact with metal pipework, for example from sinks.

Sodium azide deteriorates with time so that dehydrated media have a limited shelf-life.

    - Culture media

    - Azide glucose broth (single strength)

Beef extract						4.5g

tryptone					          15.0g

glucose						7.5g

sodium chloride (NaCl)				7.5g

sodium azide (NaN3)					0.2g

bromocresol purple

(ethanolic solution 15g/1)				1 mL

water					         up to 1000 mL

Dissolve the ingredients in the water by boiling.

Adjust the pH so that after sterilization it will be 7.2 ± 0.1 at 25 0C.

Distribute in tubes in 10 mL volumes.

Sterilize the medium for 15 min at 121 ± 1 0C.

Note - For the examination of samples of water of more than 1 mL,

double strength broth should be prepared in volumes equal to those of the

sample to be examined.

    - Bile-aesculin-azide agar

Trypone						17.0g

peptone						  3.0g

yeast extract 						  5.0g

ox-bile, dehydrated					10.0g

sodium chloride (NaCl)				  5.0g

aesculin						  1.0g

ammonium iron (III) citrate				  0.5g

sodium azide (NaN3)					0.15g

agar						      12 to 20g

water						up to 1000 mL

Dissolve the ingredients in the water by boiling.

Adjust the pH so that after sterilization it will be 7.2 ± 0.1 at 25 0C.

Distribute in suitable containers.

Sterilize for 15 min at 121 ± 1 0C.

Cool to 50 to 60 0C and pour into Petri dishes to a depth of at least 3 mm

and allow to set on a cool, horizontal surface.

    - Hydrogen peroxide, solution, 30g/l.

    - Apparatus

Usual microbiological laboratory equipment and:

9.6.7		Incubator, capable of being maintained at 35 ± 1 0C or 37 ± 1 0C.

9.6.8		Incubator, capable of being maintained at 44 ± 0.5 0C.

9.6.9		Autoclave, capable of being maintained at 121 ± 1 0C.

9.6.10		Procedure

9.6.11		Enrichment

Add 1 mL of sample (or diluted sample) to 10 mL of azide glucose broth (13.7.3.1) and mix thoroughly.

Volumes greater than 1 mL should be added to the same volume of double strength broth.

Incubate at 35 ± 1 0C for 22 ± 2 h.

Consider all tubes showing a yellow colour (throughout the whole tube or in the lower part of the tube only) as giving a positive reaction.  Reincubate negative tubes for an additional 22 ± 2 h.

After this incubation even a faint colour change to reddish purple should be considered indicative of acid production.  In order to improve the interpretation, the colour of the inoculated tube should be compared with the colour of an uninoculated tube.

For quantitative results the most probable number (MPN) method should be used.

9.6.12		Confirmation

Confirm each enrichment culture showing acid production as follows.

Steak a loopful of the resuspended enrichment broth on a plate of bile-aesculin-azide agar (13.6.4).

Incubate at 44 ± 0.5 0C for 44 ± 4 h.

Regard all plates showing a tan to black colour in the colonies and /or in the surrounding medium as giving a positive reaction.

    - Catalase test

Place a drop of hydrogen peroxide solution (13.7.4) on colonies on bile-aesculin-azide agar.  Evolution of bubbles of oxygen indicates catalase-positive organisms.  Only catalase-negative colonies should be considered as faecal streptococci.

Note - To eliminate errors due to false negative catalase reactions which may occur on the bile-aesculin-azide agar, the test may be repeated on a subculture on a non-selective medium.

9.7	DETECTION AND ENUMERATION OF FAECAL STREPTOCOCCI

#### Part 2:  METHOD BY MEMBRANE FILTRATION

    - Culture media

    - KF-streptococcus agar (Kenner)

    - Basal medium

Proteose peptone				10.0g

yeast extract					10.0g

sodium chloride (NaCl)			  5.0g

sodium glycerophosphate			10.0g

maltose					20.0g

lactose					              1.0g

sodium azide (NaN3)				  0.4g

bromocresol purple

(ethanolic solution 15/gl)			   1 mL

agar						12 to 20g

water						up to 1000 mL

9.7.4		Dissolve the ingredients in the water by heating in a boiling water-bath.

After dissolution is complete, heat for an additional 5 mins.

Allow to cool to 50 to 60 0C.

    - TTC solution (2, 3, 5-triphenyltetrazolium chloride water)

Dissolve the dye in the water by stirring.

Sterilize by filtration (0.22 µm).

The solution should be protected against the action of light.

    - Complete medium

Basal medium (13.7.3)				1000 mL

TTC solution (13.7.5)					    10 mL

Add the TTC solution to the basal medium cooled to 50 to 60 0C.  TTC is

thermolabile, so that overheating must be avoided.

Adjust the pH if necessary to 7.2 with a sterile solution of sodium

carbonate (100g/l).

Pour the medium into Petri dishes to a depth of at least 3 mm and allow to

set on a cool, horizontal surface.

Poured plates may be stored in the dark for up to 30 days at 4 ± 2 0C.

    - m-enterococcus agar (Slanetz and Barley)

    - Basal medium

tryptose						20.0g

yeast extract						  5.0g

glucose						  2.0g

dipotassium hydrogenorthophosphate

(K2 HPO4)					  4.0g

sodium azide (NaN3)					  0.4g

agar							15.0g

water					          up to 1000 mL

Dissolve the ingredients in the water by heating in a boiling water-bath.

After dissolution is complete, heat for an additional 5 min.

Cool to 50 to 600C.

    - TTC solution (See 13.7.5)

    - Complete medium

Basal medium (13.7.3)				  100 mL

TTC solution (13.7.5)					  10 mL

Add the TTC solution to the basal medium cooled to 50 to 60 0C.

Adjust the pH if necessary to 7.2 with a solution of sodium carbonate (100g/l).

Pour 20 ml into Petri dishes of 9 cm diameter (or an equivalent amount in

a dish of another size) and allow to set on a cool, horizontal surface.

Poured plates may be stored in the dark for up to 30 days at 4 ± 2 0C.

    - Bile-aesculin-azide agar

Trypone						17.0g

peptone						  3.0g

yeast extract 						  5.0g

ox-bile, dehydrated					10.0g

sodium chloride (NaCl)				  5.0g

aesculin						  1.0g

ammonium iron (III) citrate				  0.5g

sodium azide (NaN3)					0.15g

agar						      12 to 20g

water						up to 1000 mL

Dissolve the ingredients in the water by boiling.

Adjust the pH so that after sterilization it will be 7.1 ± 0.1 at 25 0C.

Distribute in volumes of 250 ml in screw-capped bottles of 500 ml capacity.

Sterilize for 15 min at 121 ± 1 0C.

Cool to 50 to 60 0C and pour into Petri dishes to a depth of at least 3 mm

and allow to set on a cool, horizontal surface.

9.7.12		Hydrogen peroxide, solution, 30g/l.

9.7.13		Apparatus

Usual microbiological laboratory equipment and:

9.7.14		Membrane filtration apparatus.

9.7.15		Sterile membrane filters, with a normal pore size of 4.45mm.

The quality of membrane filters may vary from brand to brand or even

from batch to batch.  It is therefore advisable to check the quality on a

regular basis, according to ISO 7704.

9.7.16		Incubator, capable of being maintained at 35 ± 1 0C or 37 ± 1 0C.

9.7.17		Incubator, capable of being maintained at 44 ± 0.5 0C.

9.7.18		Autoclave, capable of being maintained at 121 ± 1 0C.

9.7.19		Procedure

Filter a suitable volume of water.

Place the membrane filter on either KF-streptococcus agar (13.7.2) or

m-enterococcus agar (13.7.7).

Incubate the plates at 35 ± 1 0C or 37 ± 1 0C for 44 ± 4 h.

9.7.20		Enumeration

After incubation, count all raised colonies which show a red, maroon or pink colour, either in the center or throughout the colony.  Consider these colonies as presumptive faecal streptococci.

Note - Occasionally bacteria other than group D streptococci may produce this type of colony.  Elevation of the incubation temperature to 44 ± 0.5 0C, after an initial incubation for 5 ± 1 h at 37 ± 1 0C, may prevent the growth of these organisms.

9.7.21		Confirmation

Subculture a representative sample of typical colonies on a plate of bile-aesculin-azide agar (13.6.4)

Incubate at 44 ± 0.5 0C for 48 h.

Regard all plates showing a tan to black colour in the colonies and/or in the surrounding medium as given a positive reaction.

    - Catalase test

Place a drop of hydrogen peroxide solution (13.6.5) on colonies on bile-aesculin-azide agar (13.6.4).  Evolution of bubbles of oxygen indicates catalase-positive organisms.  Only catalase-negative colonies should be considered as faecal streptococci.

Note - To eliminate errors due to false negative catalase reactions which may occur on the vile-aesculin-azide agar, the test may be repeated on a subculture on a non-selective medium.

9.8		DETECTION AND ENUMERATION OF PSEUDOMONAS

#### AERUGINOSA

#### PART 1:  METHOD BY ENRICHMENT IN LIQUID MEDIUM

    - Dilution fluids

Use one of the diluents given in ISO 8199.

    - Culture media

It is essential that the culture medium used be suitable for the type of water to be analyzed and the purpose of the analysis.  Use the following medium for the determination of presumed Pseudomonas aeruginosa.

    - Asparagine broth with ethanol

(Drake’s medium 10)

    - Composition

###### Single

strength		Concentrated

DL-asparagine				   2 g			     3.2 g

L-proline				   1 g			     1.6 g

Anhydrous dipotassium

hydrogen phosphate		   1 g			     1.6 g

Magnesium sulfate

heptahydrate			0.5 g			      0.8 g

Anhydrous potassium

sulfate				   10 g			      16 g

Ethanol				   25 mL		       40 mL

Water					  to 1000 mL		    to 1000 mL

    - Preparation

Dissolve all the constituents in the water and proceed in either of the following ways.

Solutions to 50 0C to 55 0C, aseptically add the milk solution to the agar medium and mix well.

    - Preparation of agar plates

Distribute 15 mL portions of the final agar medium into sterile Petri dishes (see 13.18.1).  Allow the medium to solidify in the plates.  Dry the plates.  Store at 4 0C ± 1 0C for a maximum of 1 month.

    - Apparatus and glassware

Usual microbiological laboratory equipment, and:

    - Glassware

All glassware shall be sterilized at 170 0C ± 5 0C for 1 h in a dry oven or at 121 0C ± 1 0C for 15 min in an autoclave before use.

Use sterile Petri dishes with a diameter of either 90 mm or 100 mm.

9.8.9		Incubators, capable of being maintained at 37 0C ± 1 0C and 42 0C ±

0.5 0C.

9.8.10		Ultraviolet lamp emitting light of wavelength 360 nm ± 20 nm.

Note - Sterile square plastics repli dishes may be used as an alternative to glass bottles or tubes when the volume of sample or dilution of sample under examination is 1 mL or less.

Plastics Repli dishes are square divided into 25 identical compartments which can hold 1 mL of medium together with 1 mL of sample or sample dilution.  The use of these dishes allows five replicates from each of five serial dilutions of the sample to be tested simultaneously.  The dishes can be obtained presterilized.

9.8.11		Procedure

Carry out the preparation of dilutions and the most probable number technique in accordance with ISO 8199 and ISO 6887.

9.8.12		Dilutions

Prepare 10- fold serial dilutions of the sample in a presterilized diluent (13.3) in accordance with ISO 8199.

9.8.13		Inoculation

Add 1 mL from each sample, or dilution of the sample, to 4 mL portions of the medium (13.8.1) in bottles or tubes.

If larger portions of the sample (10 mL, 50 mL) or Repli dishes are to be used, add the sample to an equal volume of the concentrated medium.

9.8.14		Incubation

Incubate the containers at 37 0C ± 1 0C for 48 h.  Examine for growth and fluorescence under an ultraviolet lamp in either a darkened room or apparatus designed to exclude visible light.

Note - Incubation at 38 0C to 39 0C may be used if the water samples are likely to contain large numbers of other bacteria.  The possible adverse effect of this procedure on the number of organisms recovered should be considered.

9.8.15		Confirmation

9.8.16		Milk agar

Subculture a loopful of culture medium from each container showing either fluorescence or growth onto a milk agar plates at 42 0C ± 0.5 0C for 24 h.  Examine the plates for growth, pigment production, and casein hydrolysis (cleaning of the milk medium around the colonies) and record the reactions as shown in table 1.

Table 1 - Pseudomonas aeruginosa reactions

Typical         Atypical

Reaction mode

(1)             (2)           (3)

Casein hydrolysis					+              +              +

Growth at 42 0C ± 0.5 0C				+              +              +

Fluorescence  (under UV irradiation only)	+              +              -

Pyocyanine (blue-green) pigment			+              +              -

+ = positive reaction

- = negative reaction

Other bacteria can sometimes give atypical reactions (2) and (3).  In each

instance the procedure described in 13.8.22 should be followed.

Note - Pigment production in the culture medium may be inhibited by the growth of bacteria other than Pseudomonas aeruginosa.  In such cases, milk agar plates should be exposed to daylight at room temperature before they are examined for pigment production.

9.8.17		Enumeration

All containers of the culture medium exhibiting either growth or fluorescence, which yield colonies (after subculture on milk agar plates) that produce either reaction (1) or (2) (see table 1 in 13.8.20) shall be regarded as positive for the presence of Pseudomonas aeruginosa.

Note - Others identified as non-pigmented or atypical Pseudomonas aeruginosa

by the procedure in 13.8.22 may be included also.

9.8.18		Non-pigmented strains

Note - as a further step, it is possible to obtain confirmation of non-pigmented

strains.  If required, a suitable method is to take a loopful of culture medium and

transfer it to a milk agar plate.  The plate is incubated at a temperature of 37 0C ±

1 0C for 24 h.  A well - isolated colony is selected and final confirmation is

obtained by testing for certain biochemical characteristics.  Commercially

available identification kits may be used.

    - Expression of results

From the number of containers of culture by reference to statistical tables in ISO 8199 the most probable number of Pseudomonas aeruginosa present in 100 ml water sample in accordance with ISO 8199.

Alternatively, express the results qualitatively e.g. by stating that Pseudomonas aeruginosa were present or absent in 100 ml of water sample.

Where large volumes are examined, e.g. bottled waters, express the results

qualitatively specifying the appropriate volume.

9.9		DETECTION AND ENUMERATION OF PSEUDOMONAS

#### AERUGINOSA

#### PART 2: MEMBRANE FILTRATION METHOD

    - Drake medium 19

    - Confirmatory medium

    - Milk agar with cetrimide

Refer to section 9.8.8

    - Preparation of medium

Prepare the yeast extract broth by dissolving all the constituents in the distilled water by steaming.  Adjust the pH between 7.2 and 7.4.  Sterilize by autoclaving 121 0C ± 1 0C for 20 min.

Mix the sterile yeast extract, cetrimide and agar, and steam this mixture until the agar has dissolved.  In a separate glass container, add the skim milk powder to the distill water and mix, preferably with a magnetic stirrer, until the powder has completely dissolved.  Autoclave both

solution separately as 121 0C ± 1 0C for 5 min.  To prevent caramelization of the milk, take care to follow these instructions.  Cool the solution to

50 0C to 55 0C, aseptically add the milk solution to the agar medium and mix well.

    - Preparation of agar plates

Distribute 15 ml portions of the final agar medium into sterile Petri dishes.  Allow the medium to solidify in the plates.  Store at 4 0C ± 1 0C for a maximum of one month.

    - Apparatus and glassware

Usual microbiological laboratory equipment, and:

    - Glassware

All glassware shall be sterilized at 170 0C  ± 5 0C for 1 h in a dry oven or at 121 0C ± 1 0C for 15 min in an autoclave before use.  Use sterile Petri dishes with a diameter of either 90 mm or 100 mm.

9.9.8		Incubators, capable of being maintained at 37 0C ± 1 0C and 42 0C ±

0.5 0C.

9.9.9		Ultraviolet lamp emitting light of wavelength 360 nm ± 20 nm.

9.9.10		Dilutions

Prepare 10-fold serial dilutions of the sample in a pre-sterilized diluent.

9.9.11		Membrane Filtration

Filter volumes of the water sample or portions of the dilution through a sterile membrane filter with a rated pore diameter equivalent to 0.45 μm.  In accordance with ISO 8199, place each membrane on a sterile filter pad saturated with Drake’s medium 19, ensuring that no air is trapped beneath.

Note - Excess Drake’s medium 19 should be removed from the Petri dish prior to placing the membrane on the filter pad.

9.9.12		Incubation of membranes

Incubate the Petridishes at 37 0C ± 1 0C for 48 h in containers that prevent moisture loss.  Examine the membrane for blue-green or greenish-brown colonies, or colonies which exhibit fluorescence under exposure to ultraviolet light in either a darkened room or apparatus which exclude visible light.

Note - Incubation at 42 0C ± 0.5 0C for up to 48 h may be used if the water

samples are likely to contain large numbers of other aquatic bacteria.

The possible adverse effect of this procedure on the number of organisms

recovered should be considered.

9.9.13		Confirmation

9.9.14		Milk agar

Subculture the characteristic colonies from 13.9.12 onto the surface of milk agar plates.  Incubate the milk agar plates at 42 0C ± 0.5 0C for 24 h.  Examine the plates for growth, pigment production and casein hydrolysis cleaning of the milk medium around the colonies) and record the reactions as shown in table 1.

9.9.15		Enumeration

Count as confirmed Pseudomonas aeruginosa all colonies which exhibit the reaction (1) and (2) (see table 1 13.8.20).

Count as presumed Pseudomonas aeruginosa all colonies which show, after incubation, the following characteristics:

blue-green or greenish-brown coloration or exhibit fluorescence

when exposed to ultraviolet light.

Note - Other identified as non-pigmented or atypical Pseudomonas aeruginosa by the procedure in 13.8.22 may be included also.

9.9.16		Non-pigmented strains

Note - As a further step, it is possible to obtain confirmation of non-pigmented strains.  If required, a suitable method is to take a loopful of culture medium and transfer it to a milk agar plate.  The plate is incubated at a temperature of 37 0C ± 1 0C for 24 h.  A well-isolated colony is selected and final confirmation is obtained by testing for certain biochemical characteristics.  Commercial available identification kits may be used.

9.9.17		Expression of results

From the number of characteristic colonies counted on the membranes, and taking account of the confirmatory tests performed, calculate the number of confirmed Pseudomonas aeruginosa present in 100 ml of water sample in accordance with ISO 8199.

Alternatively, express the result qualitatively by stating that Pseudomonas aeruginosa were present or absent in 100 ml of water sample.

  - DETECTION AND ENUMERATION OF COLIFORM ORGANISMS, THERMOTOLERANT COLIFORM ORGANISMS AND PRESUMPTIVE ESCHERICHIA COLI

    - Part 1 membrane filtration method

    - Isolation media

Use one or more of the following culture media either in solid form with agar or as a broth for saturating absorbent pads.

    - Lactose TTC agar with Tergitol 72)

    - Lactose agar with Tergito1 72)

    - Membrane enriched Teepol broth2)

    - Membrane lauryl sulfate broth2)

    - Endo medium

    - LES Endo agar

    - mFC medium

    - Confirmatory media

Use one or more of the following.

    - Medium for gas production

Lactose peptone water.

    - Medium for Indole production

Tryptone water.

    - Single-tube medium for both gas and indole production

Lauryl tryptose mannitol broth with tryptophan.

    - Reagents

    - Kovacs’ reagent for Indole

    - Oxidase reagent for the Oxidadase test

    - Apparatus

Usual microbiological laboratory equipment, including:

    - Hot-air oven for dry-heat sterilization and an autoclave.

Apart from apparatus supplied sterile, glassware and other equipment shall be sterilized according to the instructions given in ISO 8199.

9.10.19	Incubator or water bath, thermostatically control at either

30 0C ± 0.5 0C

9.10.20	Incubator or water bath, thermostatically controlled at either

44 0C ± 0.25 0C or 44.5 0C ± 0.25 0C.

9.10.21	pH meter.

9.10.22	Apparatus for membrane filtration.

9.10.23	Membrane filters, usually about 47 mm or 50 mm in diameter, with filtration characteristics equalivent to a rated nominal pore diameter of 0.45 μm.  If not obtained sterile, they shall be sterilized according to the manufacture’s instructions.

9.10.24	Forceps, for handling membranes

9.10.25	Procedure

9.10.26	Preparation of the sample, filtration and Inoculation of media

For preparation of the sample, making dilutions, filtration and inoculation of isolation media, follow the instructions given in ISO 8199 and

ISO 6887.

9.10.27	For coliform organisms, filter the required volume of the sample, or a

dilution of it, through one membrane.  Place on the medium chosen, ensuring that no air is trapped underneath it.

9.10.28	For thermotolerant coliform organisms, filter the required volume of the

sample, or a dilution of it, through one membrane.  Place on the medium chosen, ensuring that no air is trapped underneath it.

Note - The volume of sample filtered should be the same as in 13.10.27.

9.10.29	Incubation of membranes

9.10.30	For coliform organisms, incubate the membrane for 18 h to 24 h at either

35 0C ± 0.5 0C or 37 0C ± 0.5 0C.

9.10.31	For thermological coliform organisms, incubate the membrane for 18 h to

24 h at either 44 0C  ± 0.25 0C or 44.5 0C ± 0.25 0C.

Notes -The same medium can generally be used for both coliform organisms and thermotolerant coliform organisms, but mFC medium should be used only at

44 0C, and Endo and LES Endo media should be used at 35 0C or 37 0C.

A preliminary period at a lower temperature such as 30 0C or the first 4 h of incubation is recommended to resuscitate stressed organisms, especially in the examination of drinking water.

9.10.32	Examination of membranes

9.10.33	Coliform organisms

Examine the membranes and count as presumptive coliform organisms all colonies, irrespective of size, which show, after incubation at 35 0C or

37 0C, the following characteristics.

–	On lactose TTC agar with Tergitol (13.10.3): a yellow, orange or brick red colouration with a yellow central halo in the medium under the membrane.

–	On Lactose agar with Tergitol 7 (13.10.4): a yellow central halo in the medium under the membrane.

–	On membrane enrich Teepol broth (13.10.5): a yellow colour extending on to the membrane.

–	On membrane lauryl sulfate broth (13.10.6): a yellow low colour extending on to the membrane.

–	On Endo agar or broth (13.10.7): a dark red colour with a golden-green metallic sheen.

–	On LES Endo agar (13.10.8): a dark red colour with a golden-green metallic sheen.

9.10.34	Thermotolerant coliform organisms

Regard as presumptive thermotolerant coliform organisms all colonies which show, after incubation at 44 0C, the same colonial characteristics

as those described in 13.10.33. with mFC medium (13.10.9), such colonies are blue in colour.

9.10.35	Confirmatory test

It is important to note that the counts of colonies on membranes at 30 0C or 37 0C and at 44 0C are only presumptive coliform results.  Since gas production is not detected, there is also an additional presumption that the organisms forming colonies can also produce gas from lactose.  For the examination of raw or partly-treated waters, this may be sufficient, but for potable supplies and other circumstances, it is important to carry out confirmatory test, preferably on pure subcultures.

9.10.36	Coliform organisms

To confirm the membrane results, subculture each colony (13.10.33) or a representative number of them to tubes of lactose peptone water (13.10.11) and incubate at 3 0C or 37 0C for 48 h: gas production within this period confirms the presence of coliform organisms.

9.10.37	Thermotolerant coliform organisms and presumptive E. coli

For thermotolerant coliform organisms and presumptive E. coli on membranes, whether incubated at 44 0C or at 35 0C subculture each colony (13.10.34) or a representative number of them, to tubes of lactose peptone water and tryptone water and incubate at 44 0C for 24 h.  Gas production in lactose peptone water confirms the presence of thermotlerant coliform

organisms, and development of a red colour at the surface of the tryptone water culture after the addition of 0.2 ml to 0.3 ml of Kovacs’ reagent (13.19.15) confirms the presence of presumptive E. coli.

Note - The use of lauryl tryptone mannitol broth with tryptophan allows both gas and indole production to be demonstrated in a single tube.

The detection of presumptive E. coli is regarded as satisfactory evidence of faecal pollution.  However, further tests for the confirmation of E. coli may be carried out if considered necessary (13.10.38).

When subculturing from colonies on the membrane to tubes of confirmatory media, it is preferable to subculture also to a plate of nutrient agar medium for the oxidase test.

9.10.38	Oxidase test

Some bacteria found in water may conform to the definition of coliform organisms in most respect, but are able to produce gas from lactose only at temperature below 37 0C.  They therefore give negative results in the standard confirmatory tests for coliform organisms and their presence in water is not usually regarded as significant.  Aeromonas species, which occur naturally in water, have an optimum growth temperature in the range 30 0C to 35 0C but may nevertheless produce acid and gas from lactose at 37 0C.  They are distinguishable from the coliform group by a positive oxidase reaction.

9.10.39	Carry out the oxidase test with pure subcultures of lactose fermenting

organisms grown on nutrient agar medium, as follows:

–     place 2 to 3 drops of freshly prepared oxidase reagent (13.19.16) on a

filter paper in a Petri dish;

–    with a glass rod, swab stick or platinum (not nichrome) wire loop,

smear some of the growth on the prepared filter paper (see note).

–    regard the appearance of a deep blue-purple colour within 10s as a

reaction.

Note - On each occasion that the oxidase reagent is used, control tests

should be conducted with cultures of an organism known to give a positive

reaction (Pseudomonas aeruginosa) and a negative reaction (E. coli).

9.10.40	Expression of results

From the number of characteristic colonies counted on the membranes and taking account of the results of the confirmatory test performed, calculate the number of coliform organisms, thermotolerant coliform organisms and presumptive E. coli present in 100 ml of the sample in accordance with ISO 8199, 13.36, according to the following equation:

A x N x Vs x F

C= ------------------

B x Vt

Where

C     is the confirmed colony count per 100 ml;

A     is the number of colonies actually confirmed;

B     is the number of colonies subcultured for confirmation;

N     is the number of characteristic colonies on the membrane

(13.10.33 and 13.10.34);

Vt     is the test volume of water sample filtered (13.10.27 and

13.10.28);

Vs     is the reference volume for expression of results (100 ml);

F      is the dilution factor.

  - DETECTION AND ENUMERATION OF COLIFORM ORGANISMS, THERMOTOLERANT COLIFORM ORGANISMS AND PRESUMPTIVE ESCHERICHIA COLI

    - Part 2 multiple tube (most probable number) method

    - Isolation media

Use one of the following culture media.

    - Lactose broth

    - Mac Conkey broth

    - Improved formate lactose glutamate medium2)

    - Lauryl tryptose (lactose) broth

    - Confirmatory media

Use one or more of the following

    - Media for gas production

    - Brilliant-green lactose (bile) broth

    - EC medium

    - Medium for indole production

Tryptone water.

    - Single-tube medium for both gas and indole production

Lauryl tryptose mannitol broth with tryptophan.

    - Reagents

    - Kovacs’ reagent for indole

    - Oxidase reagent for the oxidase test

    - Apparatus

Usual microbiological laboratory equipment, including

    - Hot-air oven for dry-heat sterilization and an autoclave.

Apart from apparatus supplied sterile, glassware and other equipment shall be sterilized according to the instruction given in ISO 8199.

    - Incubator or water bath, thermostatically controlled at either

35 0C ± 0.5 0C or 37 0C  ± 0.5 0C.

    - Incubator or water bath, thermostatically controlled at either

44 0C  ± 0.25 0C or 44.5 0C  ± 0.25 0C.

    - pH meter.

    - Procedure

    - Preparation of the sample and inoculation of media

For preparation of the sample, making dilutions and inoculation of isolation medium with test portions, follow the instructions given in

ISO 8199.  For test portions of volume greater than 5 mL, use tubes containing “double strength” isolation medium.

    - Incubation of tubes

Incubate the inoculated tubes for 48 h at either 35 0C ± 0.5 0C or

37 0C  ± 0.5 0C.

    - Examination of tubes

Examine the tube-cultures after incubation for 18 h to 24 h and regard as positive reactions those which show turbidity due to bacterial growth and gas formation in the inner inverted (Durham) tubes, together with acid production if the isolation medium contains a pH indicator.  Reincubate those tubes which do not show any or all of these changes and examine them again for positive reactions after 48 h.

    - Confirmatory Tests

It is important to note that positive reactions in tubes of isolation medium are only presumptive coliform results.  It is therefore important to carry out confirmatory tests, preferably on pure subcultures.

    - Subculture, incubation and examination

Subculture from each tube of isolation medium giving a positive result into one or more tubes of the confirmatory media (13.11.7) for gas and indole production.

Note 1 - If the least inhibitory medium (lactose broth) is used for isolation, subculture to either or the two more selective confirmatory media [brilliant-green lactose (bile) broth or EC broth] for confirmation is recommended.

9.11.27	Coliform organisms

To confirm the presence of coliform organisms, incubate one tube of

brilliant-green lactose (bile) broth (13.11.8) at either 35 0C ± 0.5 0C, and examine for gas production within 48 h.

9.11.28	Thermotolerant coliform organisms and presumptive E. coli

To confirm the presence of thermotolerant coliform organisms, incubate another tube of EC medium (13.11.8) at 44 0C for 24 h, and examine for gas production.

To confirm the presence of presumptive E. coli, incubate a tube of tryptone water (13.11.11) for indole formation at 44 0C for 24 h.  Then add 0.2 ml to 0.3 ml of Kovacs’ reagent (13.11.8) to the tube of tryptone water: development of a red colour after gentle agitation denotes the presence of indole.

Note 2 - The use of lauryl tryptose mannitol broth with tryptophan allows both gas and indole production by presumptive E. coli to be demonstrated in a single tube.

Note 3 - The detection of presumptive E. coli is regarded as satisfactory evidence of faecal pollution.  However, further tests for the confirmation of E. coli may be carried out if considered necessary (13.11.30).

Note 4 - When subculturing from colonies on the membrane to tubes of confirmatory media, it is preferable to subculture also to a plate of nutrient agar medium for the oxidase test.

9.11.29	Oxidase test (Refer to section 13.10.38)

9.11.30	Expression of results

From the number of tubes of isolation medium and confirmatory tests giving positive reactions, calculate by reference to the statistical tables in ISO 8199, the most probable numbers of coliform organisms, thermotolerant coliform organisms and presumptive E. coli in 100 mL of the sample.

9.11.31	It is recommended that the references listed in Appendix 1 be used as guides for the chemical and physical analysis of bottled water.

10.0		CONFORMITY

  - The lot or shipment sampled as in 12.1 shall be deemed to conform to this

standard if the test results satisfy the requirements of Section 5, 6, and 7, and if inspection shows that the labelling satisfies the requirements of section 10, and the average net contents of the containers is found to be not less than that declared on the label.

  - Bottled waters produced under an approved quality assurance system

shall be deemed to conform to this standard if test results obtained from routine samples taken during production satisfy the requirements specified for the relevant characteristics, and there is adequate evidence on file from testing in-plant, or by certification by the suppliers, that the materials used (containers, closures, cleaning and sanitising chemicals, chlorinating chemicals, carbon dioxide) meet the other requirements.

APPENDIX I

### RECOMMENDED METHODS OF TEST FOR PACKAGED WATER

1.    “Official Methods of Analysis,” 14th Edition, 1980, AOAC Chapter 13 of this

publication covers the analysis of water, and includes procedures for the following

components or other characteristics:-

pH; acidity; alkalinity; solids in water;

solids in solution; nitrate ion*; chloride ion*;

fluoride ion; carbonate and bicarbonate ions;

silicate ion; calcium; barium; potassium*; sulphate ion;

manganese; iodine; arsenic; bromide; lead.

Atomic absorption spectrophotometry is used for the estimation of :-

cadmium; chromium; copper; iron; lead; magnesium; manganese; mercury; silver

and zinc.

2.    “Methods of Chemical Analysis of Water and Wastes,” 1993, US Environmental

Protection Agency.

3.    “Analysis of Trihalomethanes in Drinking Water,” issued as Appendix C, Part 141,

40 Code of Federal Regulations, USA.

4.    “Methods of Organochlorine Pesticides in Industrial Effluents” issued by the US

Environmental Protection Agency, 1973.

5.    “Standard methods for the Examination of Water and Waste Water,” American

Public Health Association, 16th edition, 1985.

This includes bacteriological methods, (multiple or membrane filter), and for colour,

turbidity, odour.

6.    ISO Standard Compendium, Environmental – Water Quality, - Physical, Biological

and Microbiological Methods, International Organisation for Standardization, Vol.3,

First Edition.

7.    Schwaller, P and Schmidt-Lorenz, W in Zbl. Bakt. Hyg. Abt. C, 1981 - Method for

Ps aeruginosa and mesophilics at 42 0C.

8.    Codex Alimentarius, Volume 13.